Brief:
I'm currently working on a project to objectively track burn scar changes on a patient's face over time using 3D scans. The core problem is: how do you compare two 3D scans of the same face taken weeks apart and measure what actually changed? This week I got ICP and landmark-based registration working on the .ply files — meaning the three scans for patient 1 are now properly aligned so we can start doing real measurements.

The pipeline you built — and why each step is interesting (5 min)

Walk them through your pipeline as a technical story:

- Step 1 — MediaPipe for landmark detection: Google's MediaPipe detects 468 facial landmarks in real time. You're using just 5 key anatomical ones (sellion, pronasale, left/right tragion, menton) because those map to the Frankfurt Plane — the standard clinical reference for face alignment. The key insight: running it on a 2D render of the 3D mesh, then projecting back into 3D.

- Step 2 — ICP + Landmark registration: ICP (Iterative Closest Point) is the classic algorithm for aligning two point clouds. The issue with pure ICP on faces: it can get stuck in local minima if scans start too far apart. So you combine it with landmark-based registration first to get a coarse alignment, then refine with ICP. This two-stage approach is standard in medical image registration.

- Step 3 — Color analysis coming up: The .tif files hold the full RGB color texture. Next step is using that to detect burn-affected regions by color, then track whether the redness/discoloration is improving session by session.

What AI tools the field is using right now (5 min)

Segmentation — U-Net is still king: U-Net, designed specifically for medical image segmentation, is the most used architecture for burn wound detection. A 2025 systematic review of CNN-based burn segmentation found accuracy ranging from 76% to 99.4% across studies. One paper at MICCAI 2025 showed a new approach that's 90% more efficient than standard U-Net by not requiring large pre-labeled datasets — useful since burn data is scarce.

Burn-specific AI:

- DeepBurn (GitHub: CarstenIsert/DeepBurn) — uses Mask R-CNN to segment burn wounds from smartphone photos and estimate total body surface area burned. Good reference for where the field started.

- A 2025 paper from AIIMS Rishikesh trained on 1,003 images from 36 patients — showing it's possible to do this with relatively small clinical datasets using smart augmentation.

3D face models:

- FLAME (Faces Learned with an Articulated Model and Expressions) is the most widely used parametric 3D face model in research right now. It can be fitted to a 3D scan to get a clean, structured mesh. The GitHub repo TimoBolkart/FLAME-Universe tracks every paper and tool built on it.

- InsightFace (GitHub: deepinsight/insightface) — state-of-the-art library for 2D and 3D face analysis, landmark detection, and reconstruction. Very actively maintained, 20k+ stars. Highly relevant to your pipeline.

- Microsoft Deep3DFaceReconstruction — weakly supervised 3D face reconstruction from a single image using the Basel Face Model. Can give you a clean 3D mesh even from a 2D photo, which could be useful for cases where a full scan isn't available.

Burn Area Detection

Option 2 — Color-based pseudo-labeling (no labels needed, good for your case)

Since burn scars have a distinct reddish/discolored appearance compared to healthy skin, you can generate masks automatically using color thresholding in HSV or LAB color space. The idea:

- Convert image to HSV

- Threshold on the hue/saturation range that corresponds to redness/discoloration

- The resulting binary mask becomes your training label

You use these auto-generated masks to train a U-Net. It's not perfect but is a very common approach when labeled data is scarce — called pseudo-labeling or weak supervision.

Option 3 — Use a pre-trained model + fine-tune

Use a U-Net pre-trained on skin lesion data (e.g. trained on the ISIC dataset — melanoma segmentation, similar skin-tone color patterns) and fine-tune it on your few images. You'd still need some masks but far fewer — maybe 5–10.

What I'd recommend for your project right now:

Start with Option 2 — build the color-threshold mask generator first. It gives you:

- A working burn detector immediately (no training needed)

- Auto-generated masks you can use as training data for U-Net later

- A baseline to compare U-Net against

Here's the plan in code — want me to write it? It would be a Python script that:

- Takes your .tif face image as input

- Detects the burn/scar region using LAB color space thresholding (redness detection)

- Draws the boundary edge around it using Canny edge detection

- Outputs the mask + the original image with the burn boundary overlaid

This is exactly the edge-detection approach you mentioned earlier and would be very impressive to show today